# Regulation on Environmental Quality and Effluent Standards

Decree No. 18/2004, as amended by Decree No. 67/2010

English translation
Regulamento sobre Padrões de Qualidade Ambiental e de Emissão de Efluentes
Translated for 4P Index policy analysis — 2026-06-19

# Decree No. 67/2010 (Amendment)

of 31 December

Given the need to review and update environmental quality standards and revise applicable fees and fines, pursuant to Article 33 of the Environment Law combined with Article 6 of Decree No. 18/2004 of 2 June, the Council of Ministers decrees:

## Article 1

1. Annexes I and V referred to in Article 7 and paragraph 3 of Article 16 of the Regulation on Environmental Quality and Effluent Emission Standards, approved by Decree No. 18/2004 of 2 June, are amended and replaced by Annexes I and V to this Decree.
2. Annexes IA and IB to this Decree are approved and incorporated into the Regulation on Environmental Quality and Effluent Emission Standards.

## Article 2 — Articles 23 and 24 (as amended)

Article 23 (Special authorization issuance fees)
1. For issuance of the authorization provided in paragraph 2 of Article 22 of the Regulation on Environmental Quality and Effluent Emission Standards, a fee shall be paid by the polluter in an amount between 50,000.00 MT and 500,000.00 MT.
3. Revenue from fees collected under this Regulation shall be allocated as follows:
   a) 60% to the State Budget;
   b) 40% to the Environment Fund.

Article 24 (Violations and fines)
1. Without prejudice to other sanctions under applicable law, the following constitute violations punishable by fines between 1,000,000.00 MT and 10,000,000.00 MT:
   [subparagraphs a–c as in original decree]
2. Fines shall be graduated as follows:
   a) 1,000,000.00–2,000,000.00 MT for Category C activities under the EIA Regulation;
   b) 2,000,000.00–5,000,000.00 MT for Category B activities;
   c) 5,000,000.00–10,000,000.00 MT for Category A activities.

## Article 3 (Payment of fees and fines)

Revenues collected under this Regulation shall be paid to the competent Tax Directorate using the appropriate official form.

## Article 4 (Updating fees and fines)

The Ministers overseeing the Environment and Finance sectors shall update fee and fine amounts provided in this Regulation.

## Decree No. 18/2004 — Key provisions (base regulation, summary)

Approved 2 June 2004. Implements Article 10 of the Environment Law (Law No. 20/97).

Article 3 — Scope: Applies to all public and private activities that directly or indirectly may interfere with environmental components.

Article 7 — Sets parameters for maintenance of air quality.

Article 8 — Provides atmospheric pollutant emission standards per industrial establishment.

Article 10 — Mechanisms for atmospheric pollutant discharges through appropriate chimneys.

Article 16 — Establishes emission standards for liquid industrial effluent discharges and quality requirements before final discharge to the receiving environment.

Article 20 — Noise levels shall be established by MICOA considering the noise source, through special legislation after sector consultation.

Article 22 — Special authorization regime for emissions (referenced in amended Art. 23).

## Annex I — Air Quality Standards (as amended 2010)

Numeric limits (µg/m³) for SO₂, CO, O₃ and particulate matter across 10-minute, 15-minute, 30-minute, 1-hour, 8-hour, 24-hour and annual averaging periods (primary and secondary standards).

## Annex IA — Inorganic and Organic Carcinogenic Air Pollutants

Limits for lead, manganese, mercury, arsenic, chromium, nickel, benzene, formaldehyde, styrene, toluene, tetrachloroethylene and related substances across defined sampling periods.

## Annex IB — Substances with Odorous Properties

Concentration limits (ppm/vol) for ammonia, bromine, chlorine, methylene chloride, carbon disulfide, phenol, perchloroethylene, carbon tetrachloride and related substances.

## Annex V — Air quality, marine receptor and chemical standards (extract)

Air quality management:
• Open-air burning of solid waste, liquid waste or any other combustible material causing environmental degradation is prohibited.
• Installation and operation of domestic and industrial incinerators is prohibited, except hospital incinerators.
• Where necessary, automatic equipment measuring emitted pollutant quantities and qualities may be required.
• Emission of odorous substances perceptible beyond the emitting property boundary is prohibited; detection by credentialed agents.

Marine/ocean receiving body standards:
Pollutant levels must remain below maximum values for potentially harmful substances. Industrial and agro-livestock discharges are permitted only if:
a) Floating materials: virtually absent;
b) Oils and greases: virtually absent;
c) Substances causing colour, odour and turbidity: virtually absent;
d) Artificial colorants: virtually absent;
e) Substances forming objectionable deposits: virtually absent;
f) Substances/conditions facilitating undesirable aquatic life: virtually absent;
g) BOD₅ (20°C) ≤ 5 mg/l;
h) Dissolved oxygen ≥ 6 mg/l;
i) pH between 6.5 and 8.5; no change greater than 0.2 units from normal.

Tables I and IA set minimum/maximum concentration limits (mg/l) for metals, phenols, surfactants, organochlorine pesticides, herbicides and other chemical substances.

# 4P Index Coding Summary

Policy: Regulamento sobre Padrões de Qualidade Ambiental e de Emissão de Efluentes (Decreto n.º 18/2004, alterado pelo Decreto n.º 67/2010)
Year (last amendment): 2010
Instruments coded: 14
Full table: 4P_Index_Mozambique_Decree_18_2004.xlsx